International Cybersecurity and Digital Forensics Academy (ICDFA)

CIP-B103: Network Forensics Fundamentals

LAB 5: ARP Poisoning Forensics

Three-Week Delivery Block 2/3 | 25-26 July 2026

Prepared by: Aminu Idris, AMCPN | Founder, ICDFA

Important Pre-Lab Instruction Read the matching lecture slide deck before executing the practical. Work only inside the ICDFA-approved lab or your own isolated virtual environment. Record every command, observation and screenshot as contemporaneous evidence.

| Course Code | CIP-B103 |
| --- | --- |
| Course Title | Network Forensics Fundamentals |
| Lab Number | Lab 5 |
| Lab Title | ARP Poisoning Forensics |
| Delivery Block | 2/3 of 3 |
| Scheduled Dates | 25-26 July 2026 |
| Estimated Duration | 4-5 hours practical work plus report writing |
| Mode | Individual practical lab in an authorized virtual environment |
| Required Evidence | arp.pcap and an isolated host-only ARP capture |

# 1. Source Materials and Slide Alignment

Address Resolution Protocol Forensics: ARP operation, ARP caches, request/reply fields, capture with tshark, ARP spoofing/poisoning, man-in-the-middle consequences and evidence analysis.

The slide practical compares clean and poisoned ARP tables and uses a supplied arp.py script only within the same LAN.

The assignment requires legitimate ARP capture, spoofing capture and proof of a poisoned ARP table.

Reading Requirement Before Lab Execution The lab deliberately follows the sequence, terminology and core exercises presented in the uploaded CIP-B103 slides. Commands have been corrected where needed for Linux syntax and forensic repeatability.

# 2. Lab Scenario

A user reports that the default gateway intermittently resolves to the wrong MAC address. You will first document normal ARP resolution, then analyze a known ARP-poisoning capture. In an instructor-approved host-only network, you may reproduce a short controlled poisoning event, capture the evidence, restore the ARP mappings and explain how the attack is detected.

# 3. Learning Outcomes

Explain how ARP maps IPv4 addresses to MAC addresses on a LAN.

Inspect ARP caches on Linux and Windows systems.

Identify ARP requests, legitimate replies and unsolicited or conflicting replies.

Detect one IP address being associated with multiple MAC addresses.

Capture and document a bounded ARP-poisoning simulation in a host-only network.

Restore network state and recommend preventive controls.

# 4. Legal, Ethical and Safety Rules

Use only systems, packet captures and networks for which you have explicit authorization.

Keep the lab isolated from production, campus, office and public networks. Use NAT or host-only virtual networking as instructed.

Do not collect, disclose or reuse real credentials, personal messages or confidential traffic.

Preserve original capture files. Perform analysis on verified working copies and record SHA-256 hashes.

Stop any traffic-generation or interception process immediately after the required evidence is captured.

Restore firewall, ARP, forwarding and network settings before closing the lab.

The live component is optional and must use instructor-assigned RFC1918 addresses on a host-only/internal virtual switch. Never poison a home, school, office, hotel or public Wi-Fi network.

# 5. Required Tools and Environment

| Item | Recommended Tool | Purpose |
| --- | --- | --- |
| Virtual machines | Kali attacker/analyst plus Linux or Windows victim | Create an isolated LAN scenario. |
| Network mode | Host-only/internal network | Prevent access to production networks. |
| Capture tools | Wireshark and tshark | Capture ARP packets. |
| Packet library | python3-scapy | Run authorized training scripts. |
| System tools | ip neigh, arp, ipconfig /all | Inspect ARP tables and interfaces. |
| Evidence | arp.pcap or student capture | Analyze known poisoning indicators. |

# 6. Lab Folder Structure and Evidence Preparation

mkdir -p ~/CIP-B103-Lab5/{evidence,working,exported,reports,screenshots,scripts}
cd ~/CIP-B103-Lab5
pwd
find . -maxdepth 1 -type d -print

Create the folder structure before downloading or generating evidence. Store original captures under evidence and analysis copies under working.

sudo apt update
sudo apt install -y wireshark tshark python3-scapy net-tools
wget -O evidence/arp.pcap \
  'https://raw.githubusercontent.com/frankwxu/digital-forensics-lab/main/Networking_Forensics/lab_files/ARP_spoofing/arp.pcap'
cp --preserve=timestamps evidence/arp.pcap working/arp_working.pcap
sha256sum evidence/arp.pcap working/arp_working.pcap | tee reports/arp_capture_hashes.txt
ip -br address | tee reports/interfaces.txt
ip route | tee reports/routes.txt
ip neigh show | tee reports/arp_table_initial.txt

Screenshot required: Interface addresses, route/gateway, initial ARP table and evidence hashes.

# 7. Mini Evidence and Chain-of-Custody Worksheet

| Field | Student Entry |
| --- | --- |
| Case/lab identifier | CIP-B103-Lab5-YourFullName |
| Trainee name |  |
| Date and time started |  |
| Evidence file name(s) |  |
| Source or generation method |  |
| Original SHA-256 |  |
| Working-copy SHA-256 |  |
| Analysis workstation/VM |  |
| Notes on any changes |  |

# 8. Part A - Observe Normal ARP Resolution

Identify the victim IP, gateway IP and your interface name.

Clear only the specific training neighbor entry where permitted.

Start an ARP-only capture, ping the gateway once and stop the capture.

Identify the broadcast request and unicast reply.

IFACE=eth0
GATEWAY_IP=$(ip route | awk '/default/ {print $3; exit}')
sudo ip neigh flush "$GATEWAY_IP" dev "$IFACE"
sudo tshark -i "$IFACE" -f 'arp' -a duration:20 -w evidence/normal_arp.pcapng &
sleep 2
ping -c 1 "$GATEWAY_IP"
wait
ip neigh show | tee reports/arp_table_after_ping.txt

# 9. Part B - Analyze ARP Request and Reply Fields

tshark -r evidence/normal_arp.pcapng -Y 'arp' -T fields \
  -e frame.number -e frame.time -e eth.src -e eth.dst -e arp.opcode \
  -e arp.src.proto_ipv4 -e arp.src.hw_mac -e arp.dst.proto_ipv4 -e arp.dst.hw_mac \
  | tee reports/normal_arp_fields.tsv

| Field | Normal Request | Normal Reply |
| --- | --- | --- |
| Ethernet destination | ff:ff:ff:ff:ff:ff | Requester MAC |
| ARP opcode | 1 | 2 |
| Sender protocol address | Requester IP | Target IP |
| Sender hardware address | Requester MAC | Target MAC |
| Target protocol address | Target IP | Requester IP |
| Forensic interpretation | Who has target IP? | Target IP is at target MAC. |

# 10. Part C - Analyze the Supplied Poisoning Capture

PCAP=working/arp_working.pcap
# Inventory all ARP claims
tshark -r "$PCAP" -Y 'arp.opcode==2' -T fields \
  -e frame.number -e frame.time_epoch -e eth.src -e eth.dst \
  -e arp.src.proto_ipv4 -e arp.src.hw_mac -e arp.dst.proto_ipv4 -e arp.dst.hw_mac \
  | tee reports/arp_replies.tsv

# Summarize IP-to-MAC claims
tshark -r "$PCAP" -Y 'arp.opcode==2' -T fields -e arp.src.proto_ipv4 -e arp.src.hw_mac \
  | sort | uniq -c | sort -nr | tee reports/ip_mac_claims.txt

# Look for gratuitous or unsolicited-looking replies
tshark -r "$PCAP" -Y 'arp.opcode==2 && eth.dst!=ff:ff:ff:ff:ff:ff' -T fields \
  -e frame.number -e frame.time -e arp.src.proto_ipv4 -e arp.src.hw_mac -e eth.dst \
  | tee reports/unicast_arp_replies.tsv

# 11. Part D - Optional Controlled Host-Only Simulation

Instructor Approval Required Skip this section unless the virtual network is confirmed as host-only/internal and the victim, gateway and analyst IP addresses are assigned specifically for the lab.

Record clean ARP tables on victim and gateway.

Start ARP capture on the analyst VM.

Run the instructor-provided arp.py for no more than 30 seconds.

Capture the changed victim mapping and stop the script.

Restore correct ARP mappings or restart the isolated VMs.

# Example placeholders only - replace with instructor-assigned lab IPs
VICTIM_IP=192.168.56.20
GATEWAY_IP=192.168.56.1
IFACE=eth0

sudo tshark -i "$IFACE" -f 'arp' -a duration:40 -w evidence/controlled_arp_poison.pcapng &
sleep 2
# In another terminal, run the instructor-provided script for <=30 seconds:
# sudo timeout 30 python3 scripts/arp.py "$VICTIM_IP" "$GATEWAY_IP"
wait

ip neigh show | tee reports/analyst_arp_table_after.txt

# 12. Part E - Detection Logic and Timeline

| Time | Claimed IP | Claimed MAC | Target | Request Seen First? | Assessment |
| --- | --- | --- | --- | --- | --- |

Flag a gateway IP that changes from its legitimate MAC to the analyst/attacker MAC.

Flag repeated ARP replies without a corresponding recent request.

Flag one MAC claiming both victim and gateway IP addresses.

Correlate ARP evidence with packet forwarding, DNS/HTTP anomalies and endpoint ARP-cache changes.

# 13. Restoration and Prevention

# Stop any training script first, then clear learned entries on the lab VM
sudo ip neigh flush all
# Renew/reconnect the isolated interface as appropriate
ip neigh show

# Confirm no ARP-poisoning process remains
ps aux | grep -E '[a]rp.py|[s]capy' | tee reports/process_check.txt

Use Dynamic ARP Inspection with DHCP snooping on managed switches where available.

Segment networks, enable port security and monitor gateway IP-to-MAC consistency.

Use encrypted application protocols so interception does not expose plaintext.

Deploy ARP monitoring tools and correlate alerts with switch and endpoint logs.

# Required Screenshots Checklist

☐ Initial interface, route and ARP table.

☐ Normal broadcast ARP request.

☐ Normal ARP reply.

☐ Supplied capture hash.

☐ ARP reply inventory.

☐ IP-to-MAC claim summary.

☐ Conflicting gateway MAC evidence.

☐ Clean versus poisoned ARP table comparison.

☐ Controlled capture, if authorized.

☐ Restored ARP table and process check.

# Student Report Template

Executive summary and authorization.

Network diagram and assigned IP/MAC addresses.

Evidence integrity.

Normal ARP behaviour.

Poisoning indicators and timeline.

ARP-table comparison.

Impact assessment.

Restoration and preventive controls.

Appendix.

# Submission Requirements

One PDF report named CIP-B103-Lab5_RegNo_FullName.pdf.

The original or generated PCAP/PCAPNG evidence file and its SHA-256 hash text file.

A reports folder containing exported CSV/TXT command output and any recovered objects.

Screenshots must be readable, numbered and referenced in the report narrative.

Compress the report and evidence outputs into one ZIP named with your registration number and lab number.

Do not submit passwords or decoded credentials in public discussion channels; include them only in the protected assessment submission where required.

# Marking Rubric (100 Marks)

| Assessment Area | Marks | What the Instructor Will Check |
| --- | --- | --- |
| Safety and environment | 15 | Host-only scope and restoration documented. |
| Normal ARP analysis | 15 | Request/reply fields and ARP-cache behaviour. |
| Poisoning detection | 25 | Conflicting claims, unsolicited replies and timeline. |
| Controlled evidence or supplied PCAP analysis | 20 | Complete, repeatable capture analysis. |
| Mitigation and restoration | 15 | Correct cleanup and preventive controls. |
| Report quality | 10 | Clear diagram, screenshots and conclusions. |

# Instructor Quick Answer Guide

| Checkpoint | Expected Observation |
| --- | --- |
| ARP request opcode | 1. |
| ARP reply opcode | 2. |
| Broadcast destination | ff:ff:ff:ff:ff:ff for a typical request. |
| Poisoning indicator | An IP, especially the gateway, is claimed by a different MAC. |
| Unsolicited reply | ARP reply appears without a corresponding request. |
| MITM pattern | Attacker MAC is inserted for victim and/or gateway IP mappings. |

# Command Reference Summary

| Command | Purpose |
| --- | --- |
| ip neigh show | Display Linux neighbor/ARP cache. |
| arp -a | Display legacy ARP table. |
| arp.opcode==1 | Filter ARP requests. |
| arp.opcode==2 | Filter ARP replies. |
| arp.src.proto_ipv4 | Claimed sender IP. |
| arp.src.hw_mac | Claimed sender MAC. |

# Academic Integrity and Authorization Statement

Student Declaration I confirm that I completed this lab in an authorized environment, preserved the supplied evidence, did not target any third-party system, and accurately documented my own commands, observations and conclusions.